Bajazzerne

Techninal requirements

Crew:

Clarinet:			Per Jensen

Trompet, vocals:		Niels Mansa

Trombone:			Peter Goetz

Bass, Vocals:		Jens Kofod

Piano:				Hugo Charlé

Banjo,Vocals:       		Anders Andersen

Drums:			Flemming Lind

Soundengineer:		Robert Kofod

Equipment:

Mixerconsol Allen & Heath GL3 etc. (minimum 16/4/2 channels)

Effects: TC M2000/3000/5000 (or lexicon) reverb

Effects: TC 2290 delay

Front equalizer: KLARK DN360 or TC1128

Monitor Equalizer: TC1220/TC2240/TC1140

Frontspeakers: MT, Renkus Heinz or the like.

Monitors: MT122/MT152

Amplifiers: MT, C-Audio or the like

CD-player

Stage setup for Bajazzerne

FRONT

SPEAKERS

SPEAKERS

Piano

Trompet

Banjo

Drums

Bass

Clarinet

Trombone

1

2

7+8

5

4

3

12

11

14

13

9

MON:1

10

6

MON:4

MON:4

MON:3

MON:3

MON:2

MON:2

Technical description:

| Channel | Description | Mic | DI |
| --- | --- | --- | --- |
| 1 | Clarinet | EV Pl 80/Shure SM58 |  |
| 2 | Trompet | Shure SM58 |  |
| 3 | Trombone | EV Pl 80/Shure SM58 |  |
| 4 | Piano (left DI) | Gallien Krüger Amp | X |
|  | (Piano Right DI) |  | X |
| 5 | Bass DI | Schertler Amp | X |
| 6 | Banjo | His own pre Amp | X |
| 7 | Bass Speak | Shure Beta58A |  |
| 8 | Bass Vocal | Shure Beta58A |  |
| 9 | Trompet Vocal | Shure Beta58A |  |
| 10 | Banjo Vocal | Shure Beta58A |  |
| 11 | Bass drum | AKG D112 |  |
| 12 | Snare drum | Shure Beta 98/AKG C547 |  |
| 13 | Overhead | Shure Beta 98/AKG C418 |  |
| 14 | Overhead | Shure Beta 98/AKG C418 |  |